Audiology Practice Infection Control Protocol (example)

# Standard Precautions (All Patients)

- Perform hand hygiene before and after each patient encounter.

- Wash with soap and water (20 seconds) if visibly soiled.

- Otherwise use ≥60% alcohol-based sanitizer.

- Perform after glove removal and after device handling.

# Personal Protective Equipment (PPE)

- Routine audiometry: Gloves if ear contact expected.

- Hearing aid fitting: Gloves.

- Cerumen removal: Gloves + Mask + Eye Protection.

- Remove PPE before leaving treatment area and perform hand hygiene.

# Instrument Processing

- Non-critical items: Low-level disinfection between patients.

- Semi-critical items: High-level disinfection or disposable single-use.

- Probe tubes: Single patient use only.

- Critical instruments: Sterilization (autoclave required).

# Cerumen Management

- Gloves required; mask and eye protection recommended.

- Use disposable instruments when possible.

- Reusable instruments: Clean thoroughly then autoclave.

- Disinfect all surfaces immediately after procedure.

# Environmental Cleaning

- Disinfect chair armrests, countertops, probes, and equipment controls between patients.

- Use EPA-registered disinfectant i.e, Clorox Disinfecting Wipes (EPA Reg. No. 5813-79)

- Lysol Disinfectant Spray (EPA Reg. No. 777-99)

- CaviCide Surface Disinfectant (EPA Reg. No. 46781-6)

- Sani-Cloth Germicidal Wipes (e.g., Super Sani-Cloth, EPA Reg. No. 9480-4)

# Waste & Compliance

- Dispose blood-contaminated items in biohazard container.

- Maintain OSHA exposure control plan.

- Annual OSHA training required.

- Maintain sterilization logs if autoclave is used.